LS Contracting Inc.

27280 Smithson Valley Rd.

San Antonio, TX 78261

210-416-1844

December 1, 2023

Office of Historic Preservation City of San Antonio

1901 S. Alamo Street

San Antonio, Texas 78204

RE: PROPOSED NEW CONSTRUCTION / 107 S. PINE ST.

The attached files contain materials to assist your office in our request for a Certificate of Appropriateness for the new construction of a building at 107 S. Pine Street, existing designated Historic Landmark. The church was designated in January 2017 as part of the Eastside Churches Survey.

Currently, the original sanctuary building, now Redeemerʼs Praise Church, provides a place for religious mission, family and individual counseling, church administrative operations, and a staging area for feeding and clothing the poor, marginalized, and homeless. The Pastor and volunteers prepare meals for off-site distribution and facilitate outdoor Food Bank distributions weekly. Many of the participants to the weekly distribution come from the immediate and adjoining neighborhoods. The proposed project will be a community center to the existing church and will house all activities with the exception of religious worship services.

EXISTING HISTORIC LANDMARK – The original church on this location appears in the 1904 Sanborn Fire Map but minimal is known about the structureʼs style. Contemporaneous sources document that at least one major rebuilding campaign by the members of the Pine Street Presbyterian Church occurred in 1907 after a major fire event. Currently, 107 S. Pine is a 2483 square foot, wood frame church comprised of two volumes: the original south wing built in 1907 and the smaller north wing built sometime after 1924. Stylistic elements include Gothic revival high-pitched gable roof forms, drop lap siding, wood shingle gable infill, and exposed curved rafter tails. The standing seam metal roof on the south wing was installed over the original wood shingle roof cover sometime before the north wing was constructed. The rough framing for the doorways, gothic-inspired windows, and oculi are original but the trims are not. The east-facing main entry location is historic, and the baldachin-like wood structure may be historic, but the exterior materials are not. The south facing projecting bay contains the altar and is original.

There is an existing back doorway on the west face of the North Wing that we would like to switch with the window opening to the north on the same façade to facilitate an accessible path to the new project.

PROJECT - The 2097 square foot building is single-story building located on a corner lot among mixed uses (residential, commercial, parking, and industrial. In this case, the larger north wing contains the business operations and the modest residential scale south wing designed to fit within its development context, is a meeting room with support areas. The rear West Yard off of Omaha Street behind a gate will be used for deliveries and outgoing meals for remote distribution.

The Project is a conventional wood framed structure on raised concrete foundation with access, egress, and exit components made of concrete as well. Exterior materials include cement board lap siding for the north wing and a board and batten pattern 12 inches wide with battens 1 – ½ inch wide, for the south wing to meet, the smooth side on all cement board products will be used, with a maximum reveal of 6 – 6 inches. Aluminum clad windows with wood casing to give the appearance of wood windows, windows will be block framed to comply with OHPʼs Window Policy and include 1x window and door trims. Window trim will feature traditional dimensions and architecturally appropriate sill detail. See window detail on plans A6.0-6.2. The exterior doors to be metal clad 2-panel. The New Project entry will be on the west side of the Church. The roof for the Project will be standing seam metal to match the historic church. Standing seam metal will feature panels that are 18 to 21 inches wide, seams that are 1 – 2 inches tall, a crimped ridge seam and a standard galvalume finish. Ridges are to feature a double-munch or crimped ridge configuration; no vented ridge caps or end caps will be added. The guard- and handrails throughout the project for landings, stairs, and ramps are proposed to be light galvanized steel in an effort to minimize their visual presence.

Paint selection for exterior cement board lap siding – SW2839 and trim SW2833.

Please feel free to contact me with any questions, comments, or concerns.

Thank you,

Abe Gonzales

LS Contracting Inc.